SUPPLEMENTARY DATA

Materials and Instrumentation

The following substances were purchased from Merck: 1, 10-phenanthroline monohydrate (phen), 2, 2'-bipyridine (bpy), 2-(1H-Pyrrol-2-yl)-1H-indole (tbz) CT-DNA (Calf thymus DNA) was bought from Aldrich, and stored at 20°C. All studies used 18.2 mX ultrapure Milli-Q water. All additional chemicals and solvents came from already-established local sources. Prior to usage, all solvents underwent normal processes for purification [1]. By dissolving calculated amounts of metal complexes in DMSO and adjusting the concentrations with the right buffer, a stock solution of metal complexes was created.

A PerkinElmer 1605 Fourier transform I.R. spectrometer was utilised to record the I.R. spectra on KBr discs. Dimethyl-d6 sulfoxide (DMSO-d6) was used as the solvent and tetra methyl silane served as the internal standard to record the 1H NMR spectra on a Bruker 400-MHz spectrometer at room temperature. The UV-VISIBLE spectra were captured with a Shimadzu UV-2600 spectrophotometer. The luminescence spectrum data were recorded using a Spectrofluorometer (serial number of the Cary Eclipse instrument, MY12400004) to assess the binding constant.

Electronic absorption studies

DNA binding studies were conducted for the purpose of studying DNA-binding interactions. Double distilled water was used to prepare each solution. In Tris buffer (5mM Tris-HCL, 50 mM NaCl, pH 7.1) investigations involving the interaction of Ru(II) complex with CT-DNA were carried out. After preparation, the stock solution of CT-DNA was kept at 4º⸰ C in the dark and used within three days. Ru (II) complex stock solutions in DMSO were prepared. By treating a fixed concentration of complex (10μM) with increments (0-120μM) of the protein free DNA stock solution (0.617×10−4 M) [2], the absorption experiment for steps 1-3 in the buffer was carried out. Solutions were given 5 min to incubate before recording the complex-DNA mixture’s absorption spectra. After each measurement, UV-vis spectra were taken. After each addition of DNA solution and the intrinsic binding constant (Kb) calculated by using Equation (1) [3].

[DNA]∕(𝜀a − 𝜀f) = [DNA]∕(𝜀b − 𝜀f) + 1∕Kb(𝜀b–𝜀f)		….. (1)

εa, εb and εf stands for the apparent absorption coefficient Aobs/[complex], the extinction coefficient for the complex in its fully bound form, and, the extinction coefficient for the free complex, respectively. [DNA] is the concentration of DNA. The intrinsic binding constant is represented Kb, a graph plotted between [DNA] / (εa- εf) and [DNA]. Kb is derived using the slope-to-intercept ratio.

Fluorescence Emission

By increasing the concentration of DNA until the complex concentration was fixed, measurements of the Ru(II) complexes’ emission intensities were made. The spectra were acquired between 540nm and 760nm. Equation 2 was used to calculate the binding constant [4].

C b = Ct[(F − F0) ∕ (Fmax − F0)]				….. (2)

Where F0 is the intensity in the absence of DNA, F is the observed fluorescence emission intensity at a specific DNA concentration, and Fmax is the intensity of the maximum complex bound to DNA. Ct stands for the total complex concentration.

r∕Cf = Kb(1 – nr)						…..(3)

A graph between r/Cf versus r was used to derive the binding constant using the Scatchard equation 3, where r is the Cb/[DNA] ratio and Cf is the concentration of the free complex. The excitation wavelength for the fluorescence titration investigations was fixed at 425 nm, and the spectra were acquired by altering the pH of the solutions.

Quenching studies

In this luminescent experiment, quenching investigations with [Fe(CN)6]-4 were expanded to better understand how these complexes bind to DNA.Fluorescence quenching studies with [Fe(CN)6]-4provide some additional insight into the complexes binding to DNA. To the 3mL of the complex, the quencher(0.01M) is added in Tris HCl buffer in the presence of DNA and the absence of DNA (1:20 and 1:200, i.e., in excess) was used to execute emission quenching investigations at room temperature. Sterne Volmer equation 4 was applied to determine the quenching constant Ksv. Where Q is the concentration of the quencher, I0 and I are the fluorescence intensities in the presence and absence of the quencher, respectively, and Ksv is a linear Sterne Volmer quenching constant that may be calculated from the slope [5]

I0/I=1+KSV [Q]						…..(4)

Viscosity studies

Viscosity investigations were done by using an Ostwald viscometer which was submerged in a thermostat bath in order to maintain a consistent temperature of 30±0.10C using BPE buffer (6 mM Na2HPO4, 2 mM NaH2PO4, 1Mm Na2EDTA, pH=7.0). To reduce the complexes brought on by DNA flexibility, the 200 base pair average length CT-DNA samples were processed by sonication [6]. An average flow time was computed after each sample was repeated three times and the flow time was recorded using a digital stopwatch. The estimated information was displayed as (η/η0)1/3 vs [Ru(II)/[DNA] concentration [7], where η is the viscosity of DNA in the presence of the complex and η0is the viscosity of DNA alone. Viscosity values can be calculated from the observed ‘t’ using the flow time of DNA-containing solutions corrected for the flow time of the buffer alone (t0)[8, 9].

References

- D. Perrin, W.L.F. Annarego, D.R. Perrin. Purification of Laboratory Chemicals, 2nd Edn, Pergamon Press, New York, NY (1980).

- E.A. Steck, A.R. Day. J. Am. Chem. Soc., 65, 452 (1943). https://doi.org/10.1021/ja01243a043

- A. Wolfe, G.H. Shimer, T. Meehan. Biochem., 26, 6392 (1987). 
https://doi.org/10.1021/bi00394a013

- J.D. McGhee, P.H. von Hippel. J. Mol. Biol., 86, 469 (1974). https://doi.org/10.1016/0022-2836(74)90031-X

- Lakowicz, J. R.; Weber, G. Quenching of Fluorescence by Oxygen. A Probe for Structural Fluctuations in Macromolecules. Biochemistry. 1973, 12, 4161–4170. https://doi.org/10.1021/bi00745a020

- J.B. Chaires, N. Dattagupta, D.M. Crothers. Biochem., 21, 3927 (1982). 1982 https://doi.org/10.1021/bi00260a004

- S. Satyanarayana, J.C. Dabrowiak, J.B. Chaires. Biochem., 32, 2573 (1993).

- Satyanarayana S, Dabrowiak JC, Chaires JB (1992) Neither. DELTA.- nor.LAMBDA.-tris(phenanthroline)ruthenium(II) binds to DNA by classical intercalation. Biochemistry 31:9319–9324. https:// doi. org/ 10. 1021/ bi001 54a001

- Long EC, Barton JK (1990) On demonstrating DNA intercalation. Acc Chem Res 23(9):271–273. https:// doi. org/ 10. 1021/ ar00177a001

Figure S1. IR Spectra of Intercalator, Ligand qpd.

Figure S2. IR Spectra of [Ru(phen)2 qpd ]2+

Figure S3. IR Spectra of [Ru(bpy)2 qpd ]2+

Figure S4. IR Spectra of [Ru(tbz)2 qpd]2+

Figure S5. 1H NMR Spectra of [Ru(phen)2 qpd ]2+

Figure S6. 1H NMR Spectra of [Ru(bpy)2 qpd ]2+

Figure S7. 1H NMR Spectra of [Ru(tbz)2 qpd]2+

Figure S8. 3D model of the Ru – complexes.

- [Ru(phen)2(qpd)]+2, (2) [Ru(bpy)2(qpd)]+2, (3) [Ru(tbz)2(qpd)]+2]

Table S1. Mulliken atomic charges distribution of [Ru(A)2QDPD] (ClO4)2.2H2O complex, performed at B3LYP/ LanL2DZ basis set.

| QUEN (QDPD) | phen | bpy | tbz |
| --- | --- | --- | --- |
| - - - - N1 -0.132442 N 2 -0.132365 C3 -0.028050 C4 0.015953 C5 0.018551 C6 -0.041547 7 C 0.383051 8 C -0.040967 9 C -0.028059 10 C 0.015932 11 C 0.019291 12 C 0.380721 13 C -0.120342 14 C -0.121644 15 N -0.129716 16 N -0.128998 17 C 0.090833 18 C 0.089656 19 C -0.129770 20 C 0.145985 21 C 0.144319 22 C -0.129132 23 N -0.148471 26 N -0.147460 24 C 0.099519 25 C 0.099920 27 C -0.047224 28 C 0.024815 29 C 0.024636 30 C -0.046994 | Ru- 0.909327 N6-0.276738 N5-0.276747 N4-0.277444 N3 -0.277174 N1-0.277192 N2-0.277441 C7 -0.091266 C8 -0.091252 C9 0.565278 C10 -0.237886 C11 -0.237888 C12 0.565270 N13 0.047818 C13 0.088161 C15 0.088162 N16 0.047819 C17 -0.187585 C18 0.079549 C19 -0.187585 C20 0.079549 N21 0.019212 C22 0.049254 N23 0.019213 C24 0.049254 C25 0.040586 C26 0.079387 C27 -0.019673 C28 -0.031674 C29 0.067210 C30 -0.031674 C31 0.067210 C32 -0.019674 C33 0.036612 C34 0.084678 C35 -0.040393 C36 0.472347 C37 -0.146668 C38 -0.146401 C39 -0.040134 C40 0.046024 C41 0.036615 C42 0.472428 C43 0.038073 C44 0.085522 C45 -0.040135 C46 -0.146399 C47 -0.146666 C48 0.046240 C49 0.472348 C50 -0.040392 C51 0.084677 C52 0.046248 C54 0.472430 C55 0.046016 C56 0.085521 C57 0.038068 C58 0.040588 C59 0.079387 | Ru- 1.464861 N6-0.543732 N5-0.543723 N4 -0.542302 N3 -0.542287 N1-0.534033 N2-0.534057 C7 -0.542287 C8 0.333343 C9 0.331168 C10 -0.019137 C11 0.116902 C12 0.057539 C13 0.146257 C14 -0.018913 C15 0.117096 16 C 0.056519 17 C 0.143488 18 C 0.333372 19 C 0.331168 20 C 0.146249 21 C 0.057541 22 C -0.019136 23 C 0.116897 24 C -0.018920 25 C 0.143497 26 C 0.056517 27 C 0.117091 28 C 0.043022 29 C 0.043066 30 C 0.455048 31 C 0.128334 32 C 0.043728 33 C 0.074316 34 C 0.128324 35 C 0.043730 36 C 0.074318 37 C 0.455056 38 C -0.147994 39 C -0.148007 40 N -0.102475 41 C 0.136084 42 C 0.136098 43 N -0.102472 44 C -0.132153 45 C 0.119537 46 C -0.132155 47 C 0.119539 48 N -0.118361 49 C 0.083156 50 N -0.118362 51 C 0.083155 52 C -0.018735 53 C 0.059836 54 C 0.059836 55 C -0.018734 | Ru- 0.952675 N6-0.330042 N5-0.249082 N4 -0.339893 N3 -0.267614 N1-0.287698 N2-0.278142 C7-0.287698 C8 0.039050 C9 0.078288 C10 -0.021071 11 C -0.094007 12 C 0.572499 13 C -0.087258 14 C 0.050327 15 C 0.081107 16 C -0.026223 17 C 0.576634 18 C -0.239684 19 C -0.244384 20 N 0.048248 21 N 0.048359 22 C 0.087994 23 C 0.088582 24 C -0.187636 25 C 0.079379 26 C 0.079437 27 C -0.188193 28 N 0.019342 29 C 0.049092 30 C 0.049218 31 N 0.019092 32 C -0.031678 33 C 0.067116 34 C 0.067087 35 C -0.031881 36 C 0.135140 37 C 0.488420 38 N -0.067103 39 C 0.248983 40 C -0.031170 41 C 0.018070 42 C -0.076129 43 C 0.077786 44 C -0.310253 45 C 0.242110 46 C -0.196171 47 S 0.368538 48 C 0.144920 49 C 0.220569 50 N -0.055238 51 C 0.241945 52 C -0.036618 53 C 0.019611 54 C -0.079694 55 C 0.522782 56 C -0.129016 57 C -0.367773 58 S 0.396324 59 C 0.074930 |